| Company Name |  | Utility Services (NE) LTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location |  | UK |  |  |  |  | Project Title |  |  |  | Risk Assessment Mobile Access Platform |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk Assessment No. |  | RA.003 |  |  | Review Date |  |  |  |  | 14.11.2023 |  |  |  | Revision No. |  |  |  | 004 |  |  |  |  |  |  |  |  |  |  |
| Person Supervising Work |  | Gang Foreman |  |  |  |  | Employer |  |  |  |  |  | Utility Services (NE) LTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Persons Exposed (tick box) |  | ✓ |  | Employees |  |  |  | ✓ | Other workers |  |  |  |  |  | ✓ | Public/visitors |  |  |  |  |  | Young Persons | Estimated total number of persons at risk |  |  |  |  |  |
|  |  |  |  | New/expectant mothers |  |  |  |  |  |  |  |  |  |  |  | Vulnerable Persons |  |  |  |  |  | Other |  |  |  |  |  |  |
| Likelihood Severity Rating 1 = Very Unlikely Rating 1 = No Injury Severity Rating 2 = Unlikely Rating 2 = Minor injury or illness Rating 3 = Likely Rating 3 = 7-day injury or illness Rating 4 = Very Likely Rating 4 = Specified injury or illness Likelihood Rating 5 = Almost Certain Rating 5 = Fatality, disabling injury, and so on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Likelihood Risk = Likelihood x Severity Acceptable Medium High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard(s) | 1 |  | 2 |  |  | 3 |  |  |  |  |  | 4 |  |  |  |  | 5 |  | 6 |  | 7 |  |  | 8 |  | 9 |  |  |
|  | Factors of Harm |  |  |  |  | Risk (Likelihood x Severity) |  |  |  |  |  | Control Measures |  |  |  |  | Factor of Harm |  |  |  | Residual Risk (Likelihood x Severity) |  |  | Control measures implemented by (name) |  | Low Risk |  |  |
|  | Likelihood (1-5) |  | Severity (1-5) |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood (1-5) |  | Severity (1-5) |  |  |  |  |  |  | YES |  | NO |
| Stabilising legs not on firm, level ground. | 2 |  | 4 |  |  | 8 |  |  |  |  |  | Ground inspected prior to operation. Suitable sole plates used as required. |  |  |  |  | 1 |  | 4 |  | 4 |  |  | Gang Supervisor |  | ✓ |  |  |
| Falls from the platform | 2 |  | 5 |  |  | 10 |  |  |  |  |  | Anchorage poins checked before using Use of lanyard and full body harness as a fall arrest system |  |  |  |  | 1 |  | 5 |  | 5 |  |  | Gang Supervisor |  | ✓ |  |  |
| Brake applied to the platform whilst lifting causing ‘Swing’ | 2 |  | 3 |  |  | 6 |  |  |  |  |  | Only use slow movements on lifting, if the platform swings, cease elevation until platform is stable |  |  |  |  | 1 |  | 3 |  | 3 |  |  | Gang Supervisor |  | ✓ |  |  |

| Hazard | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Factors of Harm |  | Risk (Likelihood x Severity) | Control Measures | Factor of Harm |  | Residual Risk (Likelihood x Severity) | Control measures implemented by (name) | Acceptable Risk |  |
|  | Likelihood (1-5) | Severity (1-5) |  |  | Likelihood (1-5) | Severity (1-5) |  |  | YES | NO |
| Lone Working | 2 | 2 | 4 | Ensure that communication is available to persons working alone | 1 | 2 | 2 | Gang Supervisor | ✓ |  |
| Power failure (engine failing causing a ‘stall’ with the platform in a raised position. | 2 | 3 | 6 | Operatives trained to lower the equipment in the event of a power failure. | 1 | 3 | 3 | Gang Supervisor | ✓ |  |
| Collision from another vehicle | 3 | 5 | 15 | Work site guarded and signed to CH8. All operatives to wear full body harness attached to the vehicle via lanyard | 1 | 5 | 5 | Gang Supervisor | ✓ |  |
| Working in hours of darkness | 2 | 3 | 6 | No work should be done In hours of darkness | 1 | 3 | 3 | Gang Supervisor | ✓ |  |
| Defective Fall arrest system | 2 | 5 | 10 | Fall arrest system to be checked by operative before use. | 1 | 5 | 5 | Gang Supervisor | ✓ |  |
| Using positioning belts within the platform as a restraint | 2 | 4 | 8 | Only use Full Body Harnesses | 1 | 4 |  | Gang Supervisor | ✓ |  |
|  |  | Name |  | Position |  | Signature |  | Date |  |  |
| Persons Conducting Assessment Person Authorising Assessment |  | Chris Alexander |  | Head of HSEQ |  | C.Alexander |  | 14.11.2023 REV- 14.11.2024 |  |  |
|  |  | Justin Marshall |  | Director |  | J.Marshall |  | 14.11.2023 REV- 14.11.2024 |  |  |

This risk assessment must be communicated to operatives before work commences. A spare briefing sheet should be used and be made available to the operatives for reference