| Company Name |  | Utility Services (NE) LTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location |  | UK |  |  |  |  | Project Title |  |  |  | Risk Assessment Avoiding Danger from Over Head Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk Assessment No. |  | RA.004 |  |  | Review Date |  |  |  |  | 14.11.2023 |  |  |  | Revision No. |  |  |  | 004 |  |  |  |  |  |  |  |  |  |  |
| Person Supervising Work |  | Gang Supervisor |  |  |  |  | Employer |  |  |  |  |  | Utility Services (NE) LTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Persons Exposed (tick box) |  | ✓ |  | Employees |  |  |  | ✓ | Other workers |  |  |  |  |  | ✓ | Public/visitors |  |  |  |  |  | Young Persons | Estimated total number of persons at risk |  |  |  |  |  |
|  |  |  |  | New/expectant mothers |  |  |  |  |  |  |  |  |  |  |  | Vulnerable Persons |  |  |  |  |  | Other |  |  |  |  |  |  |
| Likelihood Severity Rating 1 = Very Unlikely Rating 1 = No Injury Severity Rating 2 = Unlikely Rating 2 = Minor injury or illness Rating 3 = Likely Rating 3 = 7-day injury or illness Rating 4 = Very Likely Rating 4 = Specified injury or illness Likelihood Rating 5 = Almost Certain Rating 5 = Fatality, disabling injury, and so on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Likelihood Risk = Likelihood x Severity Acceptable Medium High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard(s) | 1 |  | 2 |  |  | 3 |  |  |  |  |  | 4 |  |  |  |  | 5 |  | 6 |  | 7 |  |  | 8 |  | 9 |  |  |
|  | Factors of Harm |  |  |  |  | Risk (Likelihood x Severity) |  |  |  |  |  | Control Measures |  |  |  |  | Factor of Harm |  |  |  | Residual Risk (Likelihood x Severity) |  |  | Control measures implemented by (name) |  | Low Risk |  |  |
|  | Likelihood (1-5) |  | Severity (1-5) |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood (1-5) |  | Severity (1-5) |  |  |  |  |  |  | YES |  | NO |
| Overhead Un-insulated Power Lines | 3 |  | 5 |  |  | 15 |  |  |  |  |  | No part of any equipment or plant should be allowed to be within: - - 15 metres of lines suspended from steel towers - 3 metres of lines suspended from wooden poles. If the work requires equipment to work closer that stated above, the power must be isolated/re-routed and a permit-to- work obtained from the electricity supplier. If the power cannot be isolated/re-routed, Stop work and advise your supervisor. |  |  |  |  | 1 |  | 5 |  | 5 |  |  | Gang Foreman |  | ✓ |  |  |
| Uneven/Soft Ground | 2 |  | 3 |  |  | 6 |  |  |  |  |  | When judging the distance between the overhead power lines and the highest point of all plant and equipment, consideration must be given to any increases in height associated with the ground. In order to prevent lifting equipment moving or toppling over whilst working on soft ground, suitable timber pads must be placed under each stabiliser to prevent the equipment sinking. |  |  |  |  | 1 |  | 3 |  | 3 |  |  | Gang Foreman |  | ✓ |  |  |
| Direct contact with/arching of the electric current. | 3 |  | 5 |  |  | 15 |  |  |  |  |  | All equipment must be carried at waist height. Metal ladders should not be used within these areas. |  |  |  |  | 1 |  | 5 |  | 5 |  |  | Gang Foreman |  | ✓ |  |  |

| Hazard | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Factors of Harm |  | Risk (Likelihood x Severity) | Control Measures | Factor of Harm |  | Residual Risk (Likelihood x Severity) | Control measures implemented by (name) | Acceptable Risk |  |
|  | Likelihood (1-5) | Severity (1-5) |  |  | Likelihood (1-5) | Severity (1-5) |  |  | YES | NO |
| Poor visibility - Fog- low cloud - snow | 2 | 3 | 6 | Prior to commencing operations, ensure that you carryout thorough visual inspection of the working area, looking for signs of overhead equipment. If unsure due to the weather/lighting conditions stop work and advise your supervisor. | 1 | 3 | 3 | Gang Foreman | ✓ |  |
|  |  | Name |  | Position |  | Signature |  | Date |  |  |
| Persons Conducting Assessment Person Authorising the Assessment |  | Chris Alexander |  | Director |  | C.Alexander |  | 14.11.2023 REV- 14.11.2024 |  |  |
|  |  | Justin Marshall |  | Managing Director |  | J.Marshall |  | 14.11.2023 REV- 14.11.2024 |  |  |

This risk assessment must be communicated to operatives before work commences. A spare briefing sheet should be used and be made available to the operatives for reference