| Company Name |  | Utility Services (NE) LTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location |  | Nationwide |  |  |  |  | Project Title |  |  |  | Risk Assessment Rock Excavating PEU Provision and Recovery of Poles. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk Assessment No. |  | RA.008 |  |  | Review Date |  |  |  |  | 14.11.2023 |  |  |  | Revision No. |  |  |  | 004 |  |  |  |  |  |  |  |  |  |  |
| Person Supervising Work |  | Gang Supervisor |  |  |  |  | Employer |  |  |  |  |  | USL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Persons Exposed (tick box) |  | ✓ |  | Employees |  |  |  | ✓ | Other workers |  |  |  |  |  |  | Public/visitors |  |  |  |  |  | Young Persons | Estimated total number of persons at risk |  |  |  |  |  |
|  |  |  |  | New/expectant mothers |  |  |  |  |  |  |  |  |  |  |  | Vulnerable Persons |  |  |  |  |  | Other |  |  |  |  |  |  |
| Likelihood Severity Rating 1 = Very Unlikely Rating 1 = No Injury Severity Rating 2 = Unlikely Rating 2 = Minor injury or illness Rating 3 = Likely Rating 3 = 7-day injury or illness Rating 4 = Very Likely Rating 4 = Specified injury or illness Likelihood Rating 5 = Almost Certain Rating 5 = Fatality, disabling injury, and so on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Likelihood Risk = Likelihood x Severity Acceptable Medium High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard(s) | 1 |  | 2 |  |  | 3 |  |  |  |  |  | 4 |  |  |  |  | 5 |  | 6 |  | 7 |  |  | 8 |  | 9 |  |  |
|  | Factors of Harm |  |  |  |  | Risk (Likelihood x Severity) |  |  |  |  |  | Control Measures |  |  |  |  | Factor of Harm |  |  |  | Residual Risk (Likelihood x Severity) |  |  | Control measures implemented by (name) |  | Low Risk |  |  |
|  | Likelihood (1-5) |  | Severity (1-5) |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood (1-5) |  | Severity (1-5) |  |  |  |  |  |  | YES |  | NO |
| Pedestrians/vehicles | 2 |  | 3 |  |  | 6 |  |  |  |  |  | All operatives must wear HV clothing Sign and guard the working area in line with chapter 8 Use banksman whilst moving equipment in public areas Ensure public have clear access/egress |  |  |  |  | 1 |  | 3 |  | 3 |  |  | Gang Foreman |  | ✓ |  |  |
| Underground services | 3 |  | 4 |  |  | 12 |  |  |  |  |  | Read and implement the controls outlined in “Risk Assessment No 005 Avoiding Danger From Underground Services” |  |  |  |  | 1 |  | 4 |  | 4 |  |  | Gang Foreman |  | ✓ |  |  |
| Overhead services | 3 |  | 5 |  |  | 15 |  |  |  |  |  | Read and implement the controls outlined in “Risk Assessment No 004 Avoiding Danger From Overhead Services”. |  |  |  |  | 1 |  | 5 |  | 5 |  |  | Gang Foreman |  | ✓ |  |  |

| Hazard | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Factors of Harm |  | Risk (Likelihood x Severity) | Control Measures | Factor of Harm |  | Residual Risk (Likelihood x Severity) | Control measures implemented by (name) | Acceptable Risk |  |
|  | Likelihood (1-5) | Severity (1-5) |  |  | Likelihood (1-5) | Severity (1-5) |  |  | YES | NO |
| Head injury | 2 | 4 | 8 | All operatives must wear safety helmets. | 1 | 4 | 4 | Gang Foreman | ✓ |  |
| Defective equipment | 2 | 3 | 6 | The lifting equipment must be inspected weekly and the details recorded in the weekly register F91 The lifting equipment must be inspected and tested every 12 months All slings must be inspected and tested every 6 months | 1 | 3 | 3 | Gang Foreman | ✓ |  |
| Lifting operation (lowering the new pole onto the horse/placing the old pole back onto the vehicle | 2 | 4 | 8 | All operatives must stay out of the slewing area Tag lines should be used to prevent excessive movement. | 1 | 4 | 4 | Gang Foreman | ✓ |  |
| Slippery muddy wet conditions | 2 | 3 | 6 | In the event of the ground being very slippery, muddy or wet, due consideration should be given to whether this deems the operation to be safe. The Forman is to make the decision as to whether the work continues or not and to record his decision in the gang site | 1 | 3 | 3 | Gang Foreman | ✓ |  |
| Strong gusty winds | 2 | 3 | 6 | Should the working area be exposed to strong/gusty winds that make it unsafe to work, the operation MUST BE suspended. | 1 | 3 | 3 | Gang Foreman | ✓ |  |
| Working at height (attaching the rock hammer to the carne or attaching pole slings to crane). | 2 | 4 | 8 | Anything that would be assumed as working at height is to be carried out form the MEWP. In such cases where the MEWP cannot be positioned safely operatives must wear a full body harness with fall arrest. The full body harness MUST BE within calibration and a full visual check carried out before use. | 1 | 4 | 4 | Gang Foreman | ✓ |  |
|  |  | Name |  | Position |  | Signature |  | Date |  |  |
| Persons Conducting Assessment Person Authorising Assessment |  | Chris Alexander |  | Director |  | C Alexander |  | 14.11.2023 REV- 14.11.2024 |  |  |
|  |  | Justin Marshall |  | Managing Director |  | J.Marshall |  | 14.11.2023 REV- 14.11.2024 |  |  |

This risk assessment must be communicated to operatives before work commences. A spare briefing sheet should be used and be made available to the operatives for reference