| Company Name |  | Utility Services (NE) Ltd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location |  | UK |  |  |  |  | Project Title |  |  |  | Risk Assessment COSHH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk Assessment No. |  | RA.010 |  |  | Review Date |  |  |  |  | 14.11.2023 |  |  |  | Revision No. |  |  |  | 004 |  |  |  |  |  |  |  |  |  |  |
| Person Supervising Work |  | Gang Foreman |  |  |  |  | Employer |  |  |  |  |  | Utility Service (NE) LTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Persons Exposed (tick box) |  | ✓ |  | Employees |  |  |  | ✓ | Other workers |  |  |  |  |  | ✓ | Public/visitors |  |  |  |  |  | Young Persons | Estimated total number of persons at risk |  |  |  |  |  |
|  |  |  |  | New/expectant mothers |  |  |  |  |  |  |  |  |  |  |  | Vulnerable Persons |  |  |  |  |  | Other |  |  |  |  |  |  |
| Likelihood Severity Rating 1 = Very Unlikely Rating 1 = No Injury Severity Rating 2 = Unlikely Rating 2 = Minor injury or illness Rating 3 = Likely Rating 3 = 7-day injury or illness Rating 4 = Very Likely Rating 4 = Specified injury or illness Likelihood Rating 5 = Almost Certain Rating 5 = Fatality, disabling injury, and so on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Likelihood Risk = Likelihood x Severity Acceptable Medium High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard(s) | 1 |  | 2 |  |  | 3 |  |  |  |  |  | 4 |  |  |  |  | 5 |  | 6 |  | 7 |  |  | 8 |  | 9 |  |  |
|  | Factors of Harm |  |  |  |  | Risk (Likelihood x Severity) |  |  |  |  |  | Control Measures |  |  |  |  | Factor of Harm |  |  |  | Residual Risk (Likelihood x Severity) |  |  | Control measures implemented by (name) |  | Low Risk |  |  |
|  | Likelihood (1-5) |  | Severity (1-5) |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood (1-5) |  | Severity (1-5) |  |  |  |  |  |  | YES |  | NO |
| Skin Contact with Creosote | 5 |  | 2 |  |  | 10 |  |  |  |  |  | Rubber Coated Multi-Purpose gloves to be warn at all times when handling anything that has a creosote coating. Washing facilities on hand at all times. Barrier Cream distributed to all persons working near substance. |  |  |  |  | 2 |  | 2 |  | 4 |  |  | Gang Foreman |  | ✓ |  |  |
| Skin contact with swarf | 3 |  | 3 |  |  | 9 |  |  |  |  |  | Thick Protective gloves to be used at all times and where possible mechanical means of handling to be used, |  |  |  |  | 1 |  | 2 |  | 2 |  |  | Gang Foreman |  | ✓ |  |  |
| Breathing and skin contact with chemical sprays | 3 |  | 3 |  |  | 9 |  |  |  |  |  | Used on a table next to an extractor fan which is turned on and avoid spraying on self. |  |  |  |  | 1 |  | 3 |  | 3 |  |  | Workshop Supervisor |  | ✓ |  |  |

| Hazard | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Factors of Harm |  | Risk (Likelihood x Severity) | Control Measures | Factor of Harm |  | Residual Risk (Likelihood x Severity) | Control measures implemented by (name) | Acceptable Risk |  |
|  | Likelihood (1-5) | Severity (1-5) |  |  | Likelihood (1-5) | Severity (1-5) |  |  | YES | NO |
| Dust Fumes from cutting concrete | 4 | 3 | 12 | No dry cutting to be done at any time. 2nd operative to use water suppression when cutting. Dust masks to be always used . | 1 | 3 | 3 | Gang Foreman |  |  |
| Breathing and skin contact with Paints and Solvents in Workshop | 3 | 3 | 9 | Hazard Masks to be worn when using solvents. Skin Checks to be carried out. Use disposable Vinyl Gloves | 1 | 3 | 3 | Workshop Supervisor |  |  |
|  |  | Name |  | Position |  | Signature |  | Date |  |  |
| Persons Conducting Assessment Persons Authorising Assessment |  | Chris Alexander |  | Director |  | C.Alexander |  | 14.11.2023 REV- 14.11.2024 |  |  |
|  |  | Justin Marshall |  | Managing Director |  | J.Marshall |  | 14.11.2023 REV- 14.11.2024 |  |  |

This risk assessment must be communicated to operatives before work commences. A spare briefing sheet should be used and be made available to the operatives for reference