| Company Name |  | UTILITY SERVICES (NE)LTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location |  | National |  |  |  |  | Project Title |  |  |  | Works involving a PIDOC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk Assessment No. |  | RA.017 |  |  | Review Date |  |  |  |  | 14.11.2023 |  |  |  | Revision No. |  |  |  | 004. |  |  |  |  |  |  |  |  |  |  |
| Person Supervising Work |  | Team Lead |  |  |  |  | Employer |  |  |  |  |  | Utility services (NE) LTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Persons Exposed (tick box) |  | ✓ |  | Employees |  |  |  | ✓ | Other workers |  |  |  |  |  | ✓ | Public/visitors |  |  |  |  |  | Young Persons | Estimated total number of persons at risk |  |  |  |  |  |
|  |  |  |  | New/expectant mothers |  |  |  |  |  |  |  |  |  |  |  | Vulnerable Persons |  |  |  |  |  | Other |  |  |  |  |  |  |
| Likelihood Severity Rating 1 = Very Unlikely Rating 1 = No Injury Severity Rating 2 = Unlikely Rating 2 = Minor injury or illness Rating 3 = Likely Rating 3 = 7-day injury or illness Rating 4 = Very Likely Rating 4 = Specified injury or illness Likelihood Rating 5 = Almost Certain Rating 5 = Fatality, disabling injury, and so on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Likelihood Risk = Likelihood x Severity Acceptable Medium High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard(s) | 1 |  | 2 |  |  | 3 |  |  |  |  |  | 4 |  |  |  |  | 5 |  | 6 |  | 7 |  |  | 8 |  | 9 |  |  |
|  | Factors of Harm |  |  |  |  | Risk (Likelihood x Severity) |  |  |  |  |  | Control Measures |  |  |  |  | Factor of Harm |  |  |  | Residual Risk (Likelihood x Severity) |  |  | Control measures implemented by (name) |  | Low Risk |  |  |
|  | Likelihood (1-5) |  | Severity (1-5) |  |  |  |  |  |  |  |  |  |  |  |  |  | Likelihood (1-5) |  | Severity (1-5) |  |  |  |  |  |  | YES |  | NO |
| Vehicles. | 2 |  | 5 |  |  | Medium-10 |  |  |  |  |  | 1, All operatives must wear Hi-Vis Clothing 2, Sign, light and guard the work area in line with Chapter 8. 3, All vehicles to be positioned within the work site. 4, All vehicles to be fitted with an operating flashing beacon |  |  |  |  | 1 |  | 5 |  | Medium-5 |  |  | Team Leader |  | ✓ |  |  |
| Pedestrians. | 2 |  | 2 |  |  | Acceptable- 4 |  |  |  |  |  | 1, Access and egress entry points to be clearly identifiable by pedestrians. 2, All works are to STOP immediately if a pedestrian enters the work site. 3, A pedestrian marshal is to be utilised to escort pedestrians around the works site should it be required. |  |  |  |  | 1 |  | 2 |  | Acceptable 2 |  |  | Team Leader |  | ✓ |  |  |
| Overhead Services. | 2 |  | 5 |  |  | Medium- 10 |  |  |  |  |  | 1, Implement control measures from USL.HSMS.RA.004 |  |  |  |  | 1 |  | 5 |  | Medium- 5 |  |  | Team Leader |  | ✓ |  |  |

| Hazard | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Factors of Harm |  | Risk (Likelihood x Severity) | Control Measures | Factor of Harm |  | Residual Risk (Likelihood x Severity) | Control measures implemented by (name) | Acceptable Risk |  |
|  | Likelihood (1-5) | Severity (1-5) |  |  | Likelihood (1-5) | Severity (1-5) |  |  | YES | NO |
| Underground Services. | 4 | 4 | High-16 | 1, Implement control measures from USL.HSMS.RA.003 | 2 | 4 | Medium- 8 | Team Leader | ✓ |  |
| Plant/tools & equipment. | 2 | 4 | Medium-8 | 1, Where applicable all plant and equipment must carry a current LOLER certification 2, All plant must be operated by a trained operative 3, where 2 man working is applicable this MUST be adhered to 4, Where applicable daily pre-use checks must be carried out by the operator/competent person |  |  |  |  |  |  |
| PIDOC poling solution. | 2 | 4 | Medium- 8 | 1, The poling solution is to be designed by the surveyor/competent person 2, Once designed the solution must be briefed to all personnel on site, the briefing can be recorded on the POWRA 3, The poling solution is to be in line with current specifications 4, If the current specifications do not cover the poling solution e.g cannot be met as pole to badly damaged/decayed, then a safe method is to be agreed by all operatives and recorded. 5, if at any stage there is doubt as to the method STOP work and contact your line manager. 6, If an NSS (non-standard solution) is required follow control measure 5, SSRAMS/Method may be required to cover these works. | 2 | 2 | Acceptable-4 | Team leader/Line Manger | ✓ |  |
|  |  | Name |  | Position |  | Signature |  | Date |  |  |
| Persons Conducting Assessment Person Authorising the Assessment |  | Chris Alexander |  | Director |  | C.Alexander |  | 14.11.2023 REV- 14.11.2024 |  |  |
|  |  | Justin Marshall |  | Managing Director |  | J.Marshall |  | 14.11.2023 REV- 14.11.2024 |  |  |

This risk assessment must be communicated to operatives before work commences. A spate briefing sheet (GA11) should be used and be made available to the operatives for reference