| Company Name |  |  |  |  | UTILITYS SERVICES (NE) LIMITED. |  |  |  |  |  |  |  |  | Project title |  |  |  |  |  |  |  | Cabling in the Underground Network |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location |  |  |  |  |  |  |  |  |  |  |  |  |  | Contract no. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Method statement title |  |  |  |  | Cabling in the Underground Network |  |  |  |  |  |  |  |  | Reference |  |  |  |  |  |  |  | USL.HSMS.MS.010 |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This method statement has been developed and must be read with the following risk assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk Assessment reference |  |  |  |  |  |  |  |  |  |  |  |  |  | Title |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| POWRA |  |  |  |  |  |  |  |  |  |  |  |  |  | POINT OF WORK RISK ASSESSMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work commences on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Areas to be worked in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Duration of work (actual or estimated) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of workers (actual or estimated) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Personnel (include details of all personnel involved in the task and any specific training, skills or qualification required) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  |  |  |  |  | Position |  |  |  |  |  |  |  |  |  |  |  | Competence details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operative 1 |  |  |  |  |  |  |  | Gang Foreman |  |  |  |  |  |  |  |  |  |  |  | Fully trained BT Openreach Engineer & (IPAF / ALLMI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operative 2 |  |  |  |  |  |  |  | Second man |  |  |  |  |  |  |  |  |  |  |  | Fully trained BT Openreach Engineer & (IPAF / ALLMI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operative 3 (when applicable) |  |  |  |  |  |  |  | Third man |  |  |  |  |  |  |  |  |  |  |  | Fully trained BT Openreach Engineer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Emergency arrangement details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire/Evacuation |  |  |  |  | First Aid |  |  |  |  |  |  |  |  | Pollution/Spill |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire extinguishers available on all vehicles |  |  |  |  | FA Equipment available on all vehicles |  |  |  |  |  |  |  |  | Spill kits available on all vehicles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Personal protective equipment (PPE) requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other |  |  |  |  | Other |  |  |
| Head |  | Foot (BSEN180 2034:2004 |  | Hi-Vis Vest (Class 3) |  |  |  |  | Hand |  |  |  | Eye |  |  |  |  | Hearing |  |  | Fall Arrest |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ✓ |  | ✓ |  | ✓ |  |  |  |  | ✓ |  |  |  | ✓ |  |  |  |  | ✓ |  |  | ✓ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Construction Dust (note: construction dust is harmful and can lead to fatal lung diseases such as silicosis) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Will the work create dust or fumes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ✓ |  |  | Yes |  |  |  |  | No |  |
| Identify specific control measures to avoid or minimise dust or fumes |  |  |  |  |  |  |  |  |  |  |  |  |  | Any cutting equipment must be fitted with a self-contained dust suppression device. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identify respiratory protective equipment (RPE) that will be required if dust or fumes cannot be avoided |  |  |  |  |  |  |  |  |  |  |  |  |  | Minimum standard FFP3 RPE to be used when required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Materials |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Are hazardous materials to be used? |  |  |  |  |  |  |  |  |  |  | Yes | ✓ |  |  |  | No | Are COSHH/DSEAR data sheets available? |  |  |  |  |  |  |  |  | ✓ |  |  | Yes |  |  |  |  | No |
| Attach or identify specific controls required from COSHH/DSEAR risk assessment |  |  |  |  |  |  |  |  |  | COSHH SDS available on requested. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Permit Required (tick below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ✓ | Permit to work (general) |  |  |  |  |  |  |  |  | Permit to enter (confined spaces) |  |  |  |  |  |  |  |  |  |  |  |  |  | Permit to dig |  |  |  |  |  |  |  |  |  |  |
|  | Hot-work permit |  |  |  |  |  |  |  |  | Out of hours work permit |  |  |  |  |  |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |  |  |
| Details of person issuing permit |  |  |  |  |  |  | Permit to work to be issued by NWR (if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of safe method of work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SCOPE The scope of work covered by this method statement includes the methodology for cabling in duct within the underground network: PROGRAMME • Provision of the appropriate SLG for the site conditions • Set up adequate exclusion zones should be established to protect members of the public during the Cabling operation. • Partial open join box Carry out gas test • On completion of satisfactory gas test fully open joint box using the appropriate box key’s • Position cabling winch (if required) • Position cabling trailer • Position draw rope dispenser as required • Install cable as per requirements • Remove all surplus materials and leave site safe & tidy ASSOCIATED DOCUMENTS • Traffic management plan where applicable • Generic risk assessments • COSHH assessments • Point of work risk assessment • Site diary • SLG Code of practice • The appropriate craft manual • Ct Ms for working in BT Openreach underground structures • Appropriate ISIS & engineering memo’s • Job pack cabling & duct prints RESPONSIBILITIES Line managers / supervisors shall: • Ensure that all operatives have the appropriate competency to conduct the planned works. • Ensure that all correct prints service plans, line search results and safe dig information are included in Job pack. • Ensure that all operatives comply with the requirements of this method statement. • Ensure that the necessary work equipment and PPE are available to the gang. • Be the initial point of contact for the in the event of an accident / incident. • Ensure that all relevant checks are completed and the results recorded. • Ensure that all operatives have access to the current issue of the appropriate manuals & instructions. • Ensure hand cleaning facilities available onsite. • Ensure that the operatives are on-site according to the published whereabouts. • Ensure that the lead operative and all other operatives are carrying out their responsibilities listed below. • Ensure that there are the appropriate numbers of operatives are on site to carry out the work activity. • Ensure that any damage to BT or any public/private infrastructure is recorded and reported immediately to appropriate contact. • Conduct regular formal in progress checks of the operational activity. All Operatives shall: • Carry out a point of work risk assessment and ensure that all control measures are implemented in accordance with generic risk assessment and record any additional risks and required control measures. • Adhere to the requirements of this method statement. • Follow instructions given to them by the supervisor. • Ensure that all prints and services plans provided are appropriate for work area. • Ensure that a permit to dig is completed and signed. • That all pre-use checks are completed on vehicles, and ancillary equipment. • That all operatives are briefed on the method statement and sign to confirm their understanding. • That work site complies with the requirements of the relevant method statement, operator’s manuals and instructions. • The work site is safe prior to commencement of works (for both operatives and members of the public). • Any lifting activities are appropriately planned and supervised. • That each member of the gang is using the required PPE. • Visitors to site are made aware of the hazards associated with the activity being carried out before being granted entry and they are wearing the correct PPE for the site and sign the visitors’ log. • Any damage identified on site is prior to work commencing reported to the supervisor immediately. • Report any damage to BT or any public/private infrastructure is recorded and reported immediately. • Ensure that all waste and redundant plant is removed from site the worksite is left clean and tidy. TRAINING AND COMPETENCY • Personnel safety • Induction • Manual handing and kinetic lifting • Appropriate NRSWA units • Safety overhead • Underground safety • Cabling In duct – vehicle • Cabling In duct – Provide • Cable In duct light • Cable In duct heavy • Cable In duct fibre • Cable winch PLANT/MATERIAL/EQUIPMENT • An appropriate cable despising equipment • A cabling winch (where required) • Duct motors & rods (where required) • A brush & mandrel • Draw rope dispensing equipment • Sufficient road works guarding (including courtesy board with 0800 number) • Gas detection unit • Gas torch Hose & gas bottle • 110V generator • 110V air blower (for work in declassified manholes) • De-watering pump & water test kit • All appropriate hand tools • Appropriate cable socks • Box/manhole keys / lifters block & roller • Manhole lifter (for carriageway covers) • Fire extinguisher – in date and no visible damage • Water test kit (for dewatering) PERMITS • Permit to work where applicable • HA notice where applicable CONTROLS FROM RISK ASSESSMENT All control measures are contained in the generic and point of work risk assessments provided PPE Requirements • Hard hat • Safety boots (not shoes or rigger boots) • Eye shields • Gloves • Ear defenders where applicable • Dust masks where applicable • Overalls • Hi-Viz jerkin / jacket • Hi-Viz trousers where applicable EMERGENCY PROCEDURES • In the event of any accident/incident the supervisor must be informed at the earliest opportunity. • All incidents must be reported via Telsafe • The supervisor will be the main point of contact during any accident/incident. • The H&S team will be kept Informed of all accident/incidents. • A first aid kit will be available on the worksite. WORK DELIVERY Cable drums over 50kg • Are to be loaded into the vehicle by mechanical means (i.e. forklift) • No person will be permitted within the van during any such loading operation. • All such drums are to be loaded ‘flat’ and in close proximity to the side (preferred) or rear door. At destination where mechanical means may not exist, drums are to be off-loaded, within a protected worksite, by a controlled combination of sliding out/raising up from the van floor onto a suitable ground surface by a minimum of two persons positioned either side of the required cable drum, ensuring legs and feet are not at risk from the lowered cable drum. • All drums need to be appropriately secured whilst in transit. Cable drums 25kg-50kg • May be manually handled, when mechanical means are unavailable, BUT only between two or more persons. All such persons need to be ‘capable of doing so’ and trained in proper manual handling techniques. • Drum must be loaded “flat” within close proximity to the side door. • Off-loading technique must follow the same process as drums over 50kg where mechanical means may not exist, drums are to be off-loaded, within a protected worksite, by a controlled combination of sliding out/raising up from the van floor onto a suitable ground surface by a minimum of two persons positioned either side of the required cable drum, ensuring legs and feet are not at risk from the lowered cable drum. • All drums need to be appropriately secured whilst in transit. Cable drums less than 25kg • May be manually handled by any individual ‘capable of doing so’ and trained in correct manual handling techniques. • Drum should be loaded “flat” and be loaded such that all manual handling risks during off-load at destination is minimized, utilizing a controlled combination of sliding out/raising up from the van floor onto a suitable ground surface, ensuring legs and feet are not at risk from the lowered cable drum. • All drums need to be appropriately secured whilst in transit. Site set up: • Signing, lighting and guarding shall be established in accordance with the requirements of NRSWA, particular attention shall be paid to ramps, barriers and site protection in an area of high pedestrian traffic such as the vicinity of a school collages or nursing/care homes. • Adequate exclusion zones should be established to protect members of the public during the poling operation. • Underground chambers will be tested, as necessary, for explosive and asphyxiating gases. • A proprietary roding method will be employed e.g. duct motor, hand roding • A brush & mandrel will be deployed where necessary, for the cleaning & testing of ducts. • During cabling operations, existing cables will be protected or removed from the underground chambers and reformed in chambers when the cabling operation is complete. • Connection of the pulling rope to the cable, will be by means of a fused swivel connected to the cable strength member, the fuse rating being that laid down in the client specifications or as indicated by the cable manufacturer. • Sub duct; will be by means of threaded taper carrot with a swivel connector interface. • The cable/Sub duct will be lubricated by a proprietary cabling lubricant e.g. lubaduct to minimise pulling friction. • Where cable fleeting is found to be necessary, this will be carried out at a suitable location with regard to the safety of the operatives & members of the public. The area should be free from any debris that could cause damage to the cable & should be adequately guarded to prevent possible damage from third parties. The cable should be fleeted using the figure of eight methods or by the deployment of the cable-fleeting trailer. • Steady winch speeds will be maintained with stop start action minimised. • Radio contact will be maintained between the winch operator and drum operator. • No personnel will be present in underground chambers during pulling operations. • The equipment to be used will be utilised correctly and be in good condition. • Cable guides, bell mouths, rollers etc will be employed to help minimise stress on the cable during the pulling operation. • Cable end will be sealed on completion of cabling operation - All plant, equipment and materials will be removed from site. INSPECTION AND TEST All Statutory Inspection’s, test and maintenance records will be maintained in accordance with the appropriate regulations and USL procedures. APPENDIX 1 – APPLICABLE RISK ASSESSMENTS The generic task based and point of work risk assessments will be available as part of the operatives craft manual and on site documentation APPENDIX 2 – APPLICABLE COSHH ASSESSMENTS All the relevant COSHH assessments will be available on site as part of the operative’s craft manual and on site documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Details of inspections/test certificates |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEWALT 24v Handheld drill |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MAROOKER specialist tracked machine |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 WEEKLY INSPECTION ATTACHED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PALFINGER PKG7001 Crane |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LOLER CERT ATTACHED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HILUX CHERRY PICKER |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LOLER CERT ATTCAHED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GENERAL HAND TOOLS/SLINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LOLER CERTS ATATCHED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Briefing register |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A copy of this briefing register must be attached to the method statement, together with other relevant information (such as the associated risk assessments, permits or COSHH assessments). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supervisor in charge of the work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I confirm that I have read and understand the requirements of this method statement and associated risk assessments and have communicated them to operatives under my control and to those who may be affected by its requirements. Note: it is important that you test the operatives’ understanding and assume they have read and understood the method statement and risk assessments. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  | C.Alexander |  |  | Position |  |  |  | Director |  |  |  |  | Signature |  |  |  | C.Alexander |  |  |  |  | Date |  |  | 14.11.2023 |  |  |  |  |  |  |  |
| Operatives/workforce carrying out the work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I understand and will agree to adhere to the contents of this method statement and the associated risk assessments. I have attended a site induction/briefing that explained the general site rules and necessary site specific arrangements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  |  |  | Position |  |  |  |  |  |  |  |  | Signature |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |  |  |  |  |  |

If you have any doubt about information given or contained in this method statement – ask for clarification.