| Company Name |  |  |  |  | UTILITYS SERVICES (NE) LIMITED. |  |  |  |  |  |  |  |  | Project title |  |  |  |  |  |  |  | Cabling in Overhead Network |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location |  |  |  |  | National |  |  |  |  |  |  |  |  | Contract no. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Method statement title |  |  |  |  | Cabling in the Overhead Network |  |  |  |  |  |  |  |  | Reference |  |  |  |  |  |  |  | USL.HSMS.MS.011 |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This method statement has been developed and must be read with the following risk assessment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Risk Assessment reference |  |  |  |  |  |  |  |  |  |  |  |  |  | Title |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| POWRA |  |  |  |  |  |  |  |  |  |  |  |  |  | POINT OF WORK RISK ASSESSMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work commences on |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Areas to be worked in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Duration of work (actual or estimated) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of workers (actual or estimated) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Personnel (include details of all personnel involved in the task and any specific training, skills or qualification required) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  |  |  |  |  | Position |  |  |  |  |  |  |  |  |  |  |  | Competence details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operative 1 |  |  |  |  |  |  |  | Gang Foreman |  |  |  |  |  |  |  |  |  |  |  | Fully trained BT Openreach Engineer & (IPAF / ALLMI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operative 2 |  |  |  |  |  |  |  | Second man |  |  |  |  |  |  |  |  |  |  |  | Fully trained BT Openreach Engineer & (IPAF / ALLMI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operative 3 (when applicable) |  |  |  |  |  |  |  | Third man |  |  |  |  |  |  |  |  |  |  |  | Fully trained BT Openreach Engineer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Emergency arrangement details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire/Evacuation |  |  |  |  | First Aid |  |  |  |  |  |  |  |  | Pollution/Spill |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |  |  |  |  |
| Fire extinguishers available on all vehicles |  |  |  |  | FA Equipment available on all vehicles |  |  |  |  |  |  |  |  | Spill kits available on all vehicles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Personal protective equipment (PPE) requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other |  |  |  |  | Other |  |  |
| Head |  | Foot (BSEN180 2034:2004 |  | Hi-Vis Vest (Class 3) |  |  |  |  | Hand |  |  |  | Eye |  |  |  |  | Hearing |  |  | Fall Arrest |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ✓ |  | ✓ |  | ✓ |  |  |  |  | ✓ |  |  |  | ✓ |  |  |  |  | ✓ |  |  | ✓ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Construction Dust (note: construction dust is harmful and can lead to fatal lung diseases such as silicosis) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Will the work create dust or fumes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ✓ |  |  | Yes |  |  |  |  | No |  |
| Identify specific control measures to avoid or minimise dust or fumes |  |  |  |  |  |  |  |  |  |  |  |  |  | Any cutting equipment must be fitted with a self-contained dust suppression device. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identify respiratory protective equipment (RPE) that will be required if dust or fumes cannot be avoided |  |  |  |  |  |  |  |  |  |  |  |  |  | Minimum standard FFP3 RPE to be used when required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Materials |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Are hazardous materials to be used? |  |  |  |  |  |  |  |  |  |  | Yes | ✓ |  |  |  | No | Are COSHH/DSEAR data sheets available? |  |  |  |  |  |  |  |  | ✓ |  |  | Yes |  |  |  |  | No |
| Attach or identify specific controls required from COSHH/DSEAR risk assessment |  |  |  |  |  |  |  |  |  | COSHH SDS available on requested. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Permit Required (tick below) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ✓ | Permit to work (general) |  |  |  |  |  |  |  |  | Permit to enter (confined spaces) |  |  |  |  |  |  |  |  |  |  |  |  |  | Permit to dig |  |  |  |  |  |  |  |  |  |  |
|  | Hot-work permit |  |  |  |  |  |  |  |  | Out of hours work permit |  |  |  |  |  |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |  |  |
| Details of person issuing permit |  |  |  |  |  |  | Permit to work to be issued by NWR (if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of safe method of work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SCOPE The scope of work covered by this method statement includes the Installation of cable in the overhead network: PROGRAMME • Provision of the appropriate SLG for the site conditions • Set up adequate exclusion zones should be established to protect members of the public during the operation. • Position MEWP and deploy out riggers • Setting up of cable dispensing equipment • Inspection of ladders and steps • Inspection of all climbing harnesses, lanyards and working belts • Informing Bt Openreach customers of any possible loss of service where applic • Recovery of existing cable where applicable • Installation of new cable • Termination of cable • Installation lightning protection where applicable • Remove all plant & equipment leaving site clean and tidy Associated documents • Traffic management plan where applicable • Generic risk assessments • COSHH assessments • Point of work risk assessment • Site induction register • Site diary • SLG code of practice • The appropriate craft manual • Job Pack • Appropriate ISIS & engineering memo’s RESPONSIBILITIES Line managers / supervisors shall: • Ensure that all operatives have the appropriate competency to conduct the planned works • Ensure that all correct prints service plans, line search results and safe dig information are included in job pack • Ensure that all operatives comply with the requirements of this method statement. • Ensure that the necessary work equipment and PPE are available to the gang. • Be the initial point of contact for the in the event of an accident / incident. • Ensure that all relevant checks are completed and the results recorded. • Ensure that all operatives have access to the current issue of the appropriate manuals & instructions. • Ensure hand cleaning facilities available onsite. • Ensure that the operatives are on-site according to the published whereabouts. • Ensure that the lead operative and all other Operatives are carrying out their responsibilities listed below. • Ensure that there are the appropriate numbers of operatives are on site to carry out the work activity. • Ensure that any damage to BT or any public/private infrastructure is recorded and reported immediately to appropriate contact. • Conduct regular formal in progress checks of the operational activity. All operatives shall: • Carry out a point of work risk assessment and ensure that all control measures are implemented in accordance with generic risk assessment and record any additional risks and required control measures. • Adhere to the requirements of this method statement. • Follow instructions given to them by the Supervisor. • Ensure that all prints and services plans provided are appropriate for work area. • Ensure that a permit to dig is completed and signed. • That all pre-use checks are completed on vehicles, and ancillary equipment. • That all operatives are briefed on the method statement and sign to confirm their understanding. • That work site complies with the requirements of the relevant method statement, operators manuals and Instructions. • The work site is safe prior to commencement of works (for both operatives and members of the public). • Any lifting activities are appropriately planned and supervised. • That each member of the gang is using the required PPE. • Visitors to site are made aware of the hazards associated with the activity being carried out before being granted entry and they are wearing the correct PPE for the site and sign the visitors’ log. • Any damage identified on site is prior to work commencing reported to the supervisor immediately. • Report any damage to BT or any public/private infrastructure is recorded and reported immediately. • Ensure that all spoil and redundant plant is removed from site the worksite is left clean and tidy. TRAINING AND COMPETENCY • Personnel safety • MEWP operator • Induction • Manual handing and kinetic lifting • Appropriate NRSWA units • Safety overhead • Lightweight aerial cabling • Heavyweight aerial cabling • OH Cabler combined modules PLANT/MATERIAL/EQUIPMENT • An appropriate MEWP • Sufficient road works guarding (including courtesy board with 0800 number) • Ladders & steps • Ladder anti slip • Ladder stand off • Ladder tie • Chain tensioners • Appropriate cable dispensing equipment • Sash line • Tool bass • Cable, clamps, brackets & insulators • Ladder leveler • Harness (belt Safety 11) • Fire extinguisher – in date and no visible damage • All hand tools – no visible damage PERMITS • HA notice where applicable CONTROLS FROM RISK ASSESSMENT All control measures are contained in the generic and point of work risk assessments provided PPE Requirements • Hard hat • Safety boots (not shoes or rigger boots) • Eye shields • Gloves • Harness (belt Safety 11) • Overalls • Hi-Viz jerkin / jacket • Hi-Viz trousers where applicable EMERGENCY PROCEDURES • In the event of any accident/incident the Supervisor must be informed at the earliest opportunity. • All incidents must be reported via Telsafe • The supervisor will be the main point of contact during any accident/incident. • The H&S team will be kept Informed of all accident/incidents. • A first aid kit will be available on the worksite. WORK DELIVERY Site set up: • Signing, lighting and guarding shall be established in accordance with the requirements of NRSWA, Particular attention shall be paid to ramps, barriers and site protection in an area of high pedestrian traffic such as the vicinity of a school collages or nursing/care homes. • Adequate exclusion zones should be established to protect members of the public during the cabling operation. • A check must be made that all way leaves / permissions are available as necessary. • For existing cable renewals customers should be informed of temporary loss of services • MEWP correctly positioned for operations with outriggers deployed. • Inspect ladder condition before use, never climb a defective ladder • Where the pole cannot be accessed by a MEWP the pole may be climbed using an appropriate ladder and a harness with working belt and fall arrest lanyard (Safety belt 11), the ladder must be tied top & bottom and the lanyard deployed once the operative commences to climb the pole vie pole steps. • Pole test must be carried out prior to the operation, if the pole is deemed defective the work must be abandoned and a supervisor contacted • Where road crossings are required the high of existing drop wires must be checked with rods telescopic to ensure the required ground clearances can be achieved if overhead power is in the vicinity check for required clearances, if clearances cannot be achieved the work must be abandoned and a supervisor contacted • Check the proposed aerial cable route for stability, install any temporary or permanent stays as required and replace defective poles where necessary to maintain route stability • Organize tools, brackets, clamps, fixings and insulators as required in preparation for installation or removal then ascend pole and draw up cable for installation fix and tension appropriately to the required height. Securing & tensioning aerial cables • The correct type of cable must be at least 300mm from the top of the Bt Openreach pole and tensioned using the correct techniques. • Aerial cable make off’s must be 300mm apart on the pole. • Heavy weight cables must be on medium hollow poles. • Cable feeds must be correctly run a cleated using the correct size cleats for the cable, the webbing must be removed or bonded using aluminium strip. • Capping must be correctly sited and currently fastened using the correct fasteners and not obstruct the 3m marks. • Anti-oscillation twists must be inserted in aerial cable at 1 twist per 7m. • The cable/suspension steel must not interfere with the BT / Box connection lid / stay wire and must be passed around the pole the correct number of times without crossing. • The cable must be bound correctly, taking care to ensure that the binding wire does not bridge the insulator if fitted. • The correct grip suspension must be used: red – heavy aerial cable, black – lightweight aerial cable. blue – when putting a full termination at the insulator. Or if Telenko products are used the correct Telenko product must be used. • The correct type O-Rings must be used and fitted within 25mm of the end of the suspension wire. • The correct size of staples must be used and fitted with one at the rear of the pole and one either side. The staples must just be in contact with the suspension wire but not crushing it. • Insulators must be fitted correctly. • The correct type and number of aerial cable clamps must be used. • For Hollow poles the correct hollow pole collar must be used and fitted correctly. • All redundant cable and wire must be recovered using approved techniques and disposed of correctly. • On a joint user pole, use the correct method of termination / attachment. Consult your supervisor if unsure. • Terminate cable on block terminals according to the appropriate client specification install lighting protection as required in accordance with the Bt lightning protection handbook (ISIS EPT/PPS/B055 - All plant, equipment and materials will be removed from site. INSPECTION AND TEST All Statutory Inspection’s, test and maintenance records will be maintained in accordance with the appropriate regulations and USL procedures. APPENDIX 1 – APPLICABLE RISK ASSESSMENTS The generic task based and point of work risk assessments will be available as part of the operatives craft manual and on site documentation APPENDIX 2 – APPLICABLE COSHH ASSESSMENTS All the relevant COSHH assessments will be available on site as part of the operative’s craft manual and on site documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Details of inspections/test certificates |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEWALT 24v Handheld drill |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PALFINGER PKG7001 Crane |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LOLER CERT ATTACHED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HILUX CHERRY PICKER |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LOLER CERT ATTCAHED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GENERAL HAND TOOLS/SLINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  | LOLER CERTS ATATCHED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Briefing register |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A copy of this briefing register must be attached to the method statement, together with other relevant information (such as the associated risk assessments, permits or COSHH assessments). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supervisor in charge of the work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I confirm that I have read and understand the requirements of this method statement and associated risk assessments and have communicated them to operatives under my control and to those who may be affected by its requirements. Note: it is important that you test the operatives’ understanding and assume they have read and understood the method statement and risk assessments. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  | C.Alexander |  |  | Position |  |  |  | Director |  |  |  |  | Signature |  |  |  | C.Alexander |  |  |  |  | Date |  |  | 14.11.2023 |  |  |  |  |  |  |  |
| Operatives/workforce carrying out the work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I understand and will agree to adhere to the contents of this method statement and the associated risk assessments. I have attended a site induction/briefing that explained the general site rules and necessary site specific arrangements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  |  |  | Position |  |  |  |  |  |  |  |  | Signature |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |  |  |  |  |  |

If you have any doubt about information given or contained in this method statement – ask for clarification.